ULP M13: Transportation LLD

Module Low-Level Design — Multi-Region Architecture

Version 2.0

May 2026 (supersedes v1.0)

Confidential — for ULP Engineering and Operations Teams

# Change Log

| Version | Date | Author | Changes |
| --- | --- | --- | --- |
| 1.0 | 2026-04-26 | Shankar | Initial release. India-focused: trip lifecycle, GPS, geofence, POD, e-Way Bill integration. |
| 2.0 | 2026-05-XX | Shankar | MODERATE refactor (30%): Split into M13-Core (universal) + M13-IN-Plugin (e-Way Bill) + M13-US-Plugin (DOT, ELD, IFTA, HOS). All v1.0 functionality preserved. |

# Indian Client Continuity

All Indian transportation features from v1.0 remain (e-Way Bill, FASTag, FOIS, GST e-Invoice on freight, RTO permits, driver license validation per Indian RTO rules). Repackaged as M13-IN-Plugin; functionality unchanged.

# 1. Introduction

## 1.1 Module purpose

M13 (Transportation) handles trip planning, dispatch, GPS tracking, geofencing, proof of delivery (POD), and country-specific transportation compliance. Supports trucking, rail, and last-mile road transport.

## 1.2 Architecture

| Component | Scope |
| --- | --- |
| M13-Core | Trip lifecycle, dispatch, GPS, geofencing, POD, driver app, fleet management |
| M13-IN Plugin | e-Way Bill (Part A + Part B), FASTag, RTO validation, Indian driver licensing |
| M13-US Plugin | DOT compliance, ELD (Electronic Logging Device) integration, IFTA, HOS (Hours of Service), FMCSA reporting |

# 2. M13-Core: Universal

## 2.1 Trip lifecycle

Planned ──[dispatch]──▶ Dispatched ──[start]──▶ InTransit ──[stop_arrived]──▶ AtStop

│                                                                          │

│                                                                          ▼

│                                                                    [load/unload]

│                                                                          │

│                                                                          ▼

│                                                                       Departed

│                                                                          │

│                                                                          ▼

│                                                                      Completed

└─[cancel]──▶ Cancelled (anytime)

## 2.2 Universal entities

CREATE TABLE m13_trip (

id BIGINT PRIMARY KEY AUTO_INCREMENT,

tenant_id INT NOT NULL,

country_code CHAR(2) NOT NULL,

trip_number VARCHAR(50) NOT NULL,

trip_type ENUM('FTL','LTL','PARCEL','CONTAINER','BULK','RAIL') NOT NULL,

-- Vehicle & driver

vehicle_id BIGINT NOT NULL,

driver_id BIGINT NOT NULL,

co_driver_id BIGINT,                       -- US team driving common

trailer_id BIGINT,

chassis_id BIGINT,                          -- container ops (US)

-- Origin / destination

origin_address_id BIGINT NOT NULL,

destination_address_id BIGINT NOT NULL,

-- Schedule (UTC)

scheduled_start_utc DATETIME(3),

actual_start_utc DATETIME(3),

scheduled_end_utc DATETIME(3),

actual_end_utc DATETIME(3),

-- Distance / fuel

estimated_distance_km DECIMAL(10,2),

actual_distance_km DECIMAL(10,2),

unit_of_distance ENUM('KM','MI') NOT NULL DEFAULT 'KM',

-- Status

status ENUM('Planned','Dispatched','InTransit','AtStop','Departed',

'Completed','Cancelled') NOT NULL DEFAULT 'Planned',

-- Audit

created_at_utc DATETIME(3) NOT NULL,

created_by BIGINT NOT NULL,

INDEX idx_status (tenant_id, status),

INDEX idx_driver (tenant_id, driver_id, scheduled_start_utc)

);

## 2.3 Trip stops

CREATE TABLE m13_trip_stop (

id BIGINT PRIMARY KEY AUTO_INCREMENT,

tenant_id INT NOT NULL,

trip_id BIGINT NOT NULL,

sequence INT NOT NULL,

stop_type ENUM('PICKUP','DROP','WAYPOINT','FUEL','REST','BORDER') NOT NULL,

address_id BIGINT NOT NULL,

scheduled_arrival_utc DATETIME(3),

actual_arrival_utc DATETIME(3),

scheduled_departure_utc DATETIME(3),

actual_departure_utc DATETIME(3),

shipment_ids JSON,                          -- shipments handled at stop

pod_document_id BIGINT,                     -- POD if drop

notes TEXT,

INDEX idx_trip (trip_id, sequence)

);

## 2.4 GPS breadcrumbs

CREATE TABLE m13_gps_breadcrumb (

id BIGINT PRIMARY KEY AUTO_INCREMENT,

tenant_id INT NOT NULL,

trip_id BIGINT NOT NULL,

vehicle_id BIGINT NOT NULL,

recorded_at_utc DATETIME(3) NOT NULL,

latitude DECIMAL(10,7) NOT NULL,

longitude DECIMAL(10,7) NOT NULL,

speed_kph DECIMAL(6,2),

heading_deg DECIMAL(5,2),

source ENUM('DRIVER_APP','GEOTAB','TRIMBLE','KEEPTRUCKIN','SAMSARA',

'KARMA','FIELDEYE','OTHER') NOT NULL,

INDEX idx_trip_time (trip_id, recorded_at_utc)

) PARTITION BY RANGE (recorded_at_utc) (...);

# 3. Driver App (Mobile)

- Built on Kotlin Multiplatform Mobile (KMP) for Android + iOS

- Universal features: trip list, trip details, navigation, status updates, POD upload, fuel logs

- Country plugin overlays: HOS clock for US drivers; e-Way Bill update for India drivers

- Offline-first: works without connectivity; syncs when online

- Battery-conscious GPS: adaptive sampling based on motion detection

## 3.1 POD capture

- Photo capture (with metadata: GPS, timestamp)

- Signature capture (touchscreen)

- Damage report with photo

- OCR-based BOL number capture

- Auto-upload when on Wi-Fi (configurable)

# 4. Telematics Integration

| Provider | Region | Capabilities |
| --- | --- | --- |
| Geotab | Global | GPS, fuel, ignition, accelerometer |
| Trimble | Global | GPS, fleet management, ELD |
| KeepTruckin / Motive | US primary | ELD, GPS, dashcam |
| Samsara | US primary | ELD, GPS, AI dashcam, sensor |
| Karma / FieldEye | India | GPS, fuel sensor, asset tracking |
| LocoNav | India | GPS, dashcam, fleet |
| Direct vehicle OEM (Tesla, Volvo Connect) | Global | OEM telematics |

All providers integrated via ITelematicsProvider abstraction; country-agnostic.

# 5. M13-IN: India Plugin

## 5.1 e-Way Bill

- Generate Part A on shipment dispatch (consignment data)

- Generate Part B on driver/vehicle assignment

- NIC EWB API integration

- Validity: 1 day per 200 km

- Extension via NIC API

- Cancellation within 24 hours

- QR code for inspection

## 5.2 Indian transportation compliance

- FASTag integration for tolls

- FOIS (Freight Operations Information System) for rail

- GST e-Invoice on freight bills (cross-ref M17-IN)

- RTO permits validation per route

- Driver license validation per Indian RTO rules

- PUC (Pollution Under Control) certificate validation

- Vehicle Fitness Certificate (FC) validation

## 5.3 Indian-specific data

CREATE TABLE m13in_ewb (

id BIGINT PRIMARY KEY AUTO_INCREMENT,

tenant_id INT NOT NULL,

trip_id BIGINT NOT NULL,

shipment_id BIGINT,

ewb_number VARCHAR(20),                     -- 12-digit number

ewb_date DATETIME(3),

valid_until DATETIME(3),

generation_status ENUM('Pending','Generated','Cancelled','Expired','Extended') NOT NULL,

cancel_reason VARCHAR(255),

ewb_part_a_data JSON,

ewb_part_b_data JSON,

qr_code_url VARCHAR(500),

INDEX idx_trip (trip_id)

);

# 6. M13-US: United States Plugin

## 6.1 DOT (Department of Transportation) compliance

- USDOT Number tracked per carrier

- MC Number (Motor Carrier authority) tracked

- Operating authority status verification (FMCSA SAFER)

- Insurance compliance: minimum $750K liability for general freight, $1M for hazmat

## 6.2 ELD (Electronic Logging Device) mandate

Per FMCSA, commercial motor vehicles must use ELDs to track Hours of Service. M13-US integrates with major ELD providers (Geotab, KeepTruckin/Motive, Samsara) to automatically capture driver duty status.

- ELD integration via provider APIs (real-time)

- Duty status: Driving, On-Duty Not Driving, Sleeper Berth, Off-Duty

- Auto-detect engine on/off, vehicle in motion

- RODS (Record of Duty Status) generated automatically

- Driver edit support with audit trail

- AOBRD legacy support phased out

## 6.3 HOS (Hours of Service) rules

| Rule | Limit |
| --- | --- |
| 11-hour driving limit | Max 11 hours driving after 10 consecutive hours off-duty |
| 14-hour limit | May not drive beyond 14th hour after coming on-duty |
| 30-minute rest break | Required after 8 cumulative hours of driving |
| 60/70-hour limit | Max 60 hours / 7 days OR 70 hours / 8 days |
| 34-hour restart | Reset 60/70 limit with 34 consecutive hours off-duty |
| Sleeper berth provision | Drivers may split 10-hour off-duty as 7+3 or 8+2 |
| Adverse driving conditions | Up to 2 additional hours allowed in adverse weather |
| Short-haul exception | 150 air-mile radius, return to same location, paper logs OK |

## 6.4 IFTA (International Fuel Tax Agreement)

Carriers operating across US states + Canadian provinces must report fuel consumption and pay/claim fuel taxes through IFTA.

- Track miles driven per jurisdiction

- Track fuel purchases per jurisdiction

- Quarterly IFTA return preparation (compute net tax due/refund)

- IFTA decals tracked per vehicle

- Trip permit support for non-IFTA jurisdictions

## 6.5 US-specific data

CREATE TABLE m13us_hos_log (

id BIGINT PRIMARY KEY AUTO_INCREMENT,

tenant_id INT NOT NULL,

driver_id BIGINT NOT NULL,

log_date DATE NOT NULL,                     -- in driver's local time

duty_status ENUM('OFF_DUTY','SLEEPER_BERTH','DRIVING','ON_DUTY_NOT_DRIVING') NOT NULL,

start_time_utc DATETIME(3) NOT NULL,

end_time_utc DATETIME(3),

vehicle_id BIGINT,

miles_driven DECIMAL(8,2),

source ENUM('ELD','DRIVER_EDIT','AUTO_GENERATED') NOT NULL,

edited_by BIGINT,                            -- if driver edit

edit_reason VARCHAR(255),

INDEX idx_driver_date (driver_id, log_date)

);

CREATE TABLE m13us_ifta_record (

id BIGINT PRIMARY KEY AUTO_INCREMENT,

tenant_id INT NOT NULL,

vehicle_id BIGINT NOT NULL,

jurisdiction VARCHAR(20) NOT NULL,           -- "CA", "NV", "AB" (Alberta), etc.

record_date DATE NOT NULL,

miles DECIMAL(10,2) NOT NULL,

fuel_gallons DECIMAL(10,3),

tax_paid_gallons DECIMAL(10,3),              -- fuel purchased in jurisdiction

source ENUM('TELEMATICS','FUEL_RECEIPT','MANUAL') NOT NULL,

INDEX idx_jurisdiction_date (jurisdiction, record_date)

);

# 7. Geofencing

- Universal geofence definition: polygon coordinates

- Predefined zones: customer locations, warehouses, fuel stations, depots

- Triggered events: ENTER, EXIT, DWELL_TIMEOUT

- Notifications: customer arrival alerts, geo-fence-based status auto-update

- Used for: ETA computation, customer notifications, driver attendance

# 8. Calendar Dispatch

- Visual dispatch board: rows = drivers/trucks/chassis, columns = time slots

- Drag-and-drop assignment

- Conflict detection: overlapping trips, HOS violations (US), license/permit gaps (India)

- Multi-day view

- Shift-and-team support (team driving for US long-haul)

- Real-time refresh as trips progress

# 9. Reports

- Trip-level: planned vs actual time, miles, fuel

- Driver-level: HOS compliance (US), trip count, on-time %, fuel efficiency

- Vehicle-level: utilization, mileage, fuel economy, maintenance schedule

- Customer-level: on-time delivery %, exception count, transit time

- Lane-level: profitability, capacity utilization

- Country-specific: e-Way Bill compliance % (India), IFTA quarterly summary (US), HOS violation report (US)

# 10. Migration from v1.0

- Add country_code column to m13_trip, m13_vehicle, m13_driver

- Existing m13in_ewb table unchanged

- Existing v1.0 endpoints continue to work for India tenants

- US tenants gain access to new endpoints when M13-US plugin loaded

- No data migration; only schema additions

— End of M13 v2.0 LLD —